AutoCAD CADmep software users, please be aware of the following software limitations that currently exist when using our highly requested flexible hoses and fire sprinkler ceiling bracket design files:

- Upon taking off any of the FlexHose ITMs, due to the CID’s limitations, the software will automatically select the first size from the product list even if other sizes were selected.

To assure that the correct size/length is selected, double click on the FlexHose in model space after the take-off and re-select the desired flex size and length. Then, the FlexHose will be adjusted accordingly.

Steps to complete brackets hoses assembly:

- Add the required Brackets and or FlexHoses from the Database items folders to the service file.

- For complete assembly, place the desired bracket in model space as shown by clicking on the bracket’s ITM from the service file.

- Note: Upon Take-off, all brackets offer users two sizes, accounting for the different sizes of sprinkler heads that may be attached to the pre-embedded drop reducer.

- Size selections are ½” or ¾” in diameter.

- After placing the bracket in the proper location, click on the desired FlexHose, select any size from the product list. (This won’t be the final selection due to the software limitation note mentioned above.)

- Select the connection point on the Tee, Outlet or piping fitting first as “END 1” then select the bracket’s connection point as “END 2” then press Enter.

To assure that the correct size/length is being used, double click on the FlexHose in model space and re-select the desired flex size and length from the ITM’s product list.

For more support and information, contact ASC-ES at AskASC@asc-es.com